SIMULATION EXPERIMENT – 1

1.Design, Realization and Performance Analysis of Butterworth IIR Low-Pass and High-Pass Digital Filters Using MATLAB

code :

clc;

clear;

close all;

% Butterworth IIR Low-Pass and High-Pass Filter Design

% Input specifications

Fs = 1000;       % Sampling frequency in Hz

Fp_LP = 100;     % Low-pass passband frequency

Fst_LP = 150;    % Low-pass stopband frequency

Fst_HP = 200;    % High-pass stopband frequency

Fp_HP = 250;     % High-pass passband frequency

Ap = 1;          % Passband attenuation in dB

Ast = 40;        % Stopband attenuation in dB

%% LOW-PASS FILTER DESIGN

Wp_LP = Fp_LP/(Fs/2);

Ws_LP = Fst_LP/(Fs/2);

[nLP, WnLP] = buttord(Wp_LP, Ws_LP, Ap, Ast);

[bLP, aLP] = butter(nLP, WnLP, 'low');

disp('LOW-PASS FILTER');

disp('Filter Order:');

disp(nLP);

disp('Numerator Coefficients:');

disp(bLP);

disp('Denominator Coefficients:');

disp(aLP);

[HLP, fLP] = freqz(bLP, aLP, 1024, Fs);

figure;

subplot(2,1,1);

plot(fLP, 20*log10(abs(HLP)+eps));

grid on;

title('Butterworth Low-Pass Filter - Magnitude Response');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

subplot(2,1,2);

plot(fLP, unwrap(angle(HLP)));

grid on;

title('Low-Pass Filter - Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

%% HIGH-PASS FILTER DESIGN

Wp_HP = Fp_HP/(Fs/2);

Ws_HP = Fst_HP/(Fs/2);

[nHP, WnHP] = buttord(Wp_HP, Ws_HP, Ap, Ast);

[bHP, aHP] = butter(nHP, WnHP, 'high');

disp('HIGH-PASS FILTER');

disp('Filter Order:');

disp(nHP);

disp('Numerator Coefficients:');

disp(bHP);

disp('Denominator Coefficients:');

disp(aHP);

[HHP, fHP] = freqz(bHP, aHP, 1024, Fs);

figure;

subplot(2,1,1);

plot(fHP, 20*log10(abs(HHP)+eps));

grid on;

title('Butterworth High-Pass Filter - Magnitude Response');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

subplot(2,1,2);

plot(fHP, unwrap(angle(HHP)));

grid on;

title('High-Pass Filter - Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

%% POLE-ZERO AND STABILITY ANALYSIS

figure;

zplane(bLP, aLP);

title('Low-Pass Filter Pole-Zero Plot');

figure;

zplane(bHP, aHP);

title('High-Pass Filter Pole-Zero Plot');

disp('Both filters designed successfully.');

OUTPUT: