SIMULATION EXPERIMENT – 1

Design and Performance Analysis of Chebyshev Type-I IIR Low-Pass and High-Pass Filters Using MATLAB

code:

clc;

clear;

close all;

% Input Parameters

Fs = 1000;      % Sampling frequency in Hz

Fp = 100;       % Passband edge frequency in Hz

Fst = 150;      % Stopband edge frequency in Hz

Rp = 1;         % Passband ripple in dB

Rs = 40;        % Stopband attenuation in dB

% Normalize Frequencies

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

% Chebyshev Type-I Low-Pass Filter

[Nlp, Wnlp] = cheb1ord(Wp, Ws, Rp, Rs);

[blp, alp] = cheby1(Nlp, Rp, Wnlp, 'low');

% Chebyshev Type-I High-Pass Filter

[Nhp, Wnhp] = cheb1ord(Ws, Wp, Rp, Rs);

[bhp, ahp] = cheby1(Nhp, Rp, Wnhp, 'high');

% Butterworth Low-Pass Filter

[Nb, Wnb] = buttord(Wp, Ws, Rp, Rs);

[bb, ab] = butter(Nb, Wnb, 'low');

% Display Filter Orders and Coefficients

disp('Chebyshev Low-Pass Filter Order:');

disp(Nlp);

disp('Low-Pass Numerator Coefficients:');

disp(blp);

disp('Low-Pass Denominator Coefficients:');

disp(alp);

disp('Chebyshev High-Pass Filter Order:');

disp(Nhp);

disp('High-Pass Numerator Coefficients:');

disp(bhp);

disp('High-Pass Denominator Coefficients:');

disp(ahp);

disp('Butterworth Low-Pass Filter Order:');

disp(Nb);

% Frequency Response

[Hlp, f1] = freqz(blp, alp, 1024, Fs);

[Hhp, f2] = freqz(bhp, ahp, 1024, Fs);

[Hb, f3] = freqz(bb, ab, 1024, Fs);

% Plot Magnitude Responses

figure;

plot(f1, 20*log10(abs(Hlp)+eps), 'b', 'LineWidth', 1.5);

hold on;

plot(f2, 20*log10(abs(Hhp)+eps), 'r', 'LineWidth', 1.5);

plot(f3, 20*log10(abs(Hb)+eps), 'k--', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response of IIR Filters');

legend('Chebyshev Low-Pass', ...

'Chebyshev High-Pass', ...

'Butterworth Low-Pass');

ylim([-80 5]);

% Plot Phase Responses

figure;

plot(f1, unwrap(angle(Hlp)), 'b', 'LineWidth', 1.5);

hold on;

plot(f2, unwrap(angle(Hhp)), 'r', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Radians)');

title('Phase Response of Chebyshev Type-I Filters');

legend('Low-Pass', 'High-Pass');

% Display Frequency Response

disp('Filter design and analysis completed successfully.');

OUTPUT: